| NOS Title | NOS Reference and New Version | Grid version control | Edit date | Edits by |
| --- | --- | --- | --- | --- |
| Apply solid render to background surfaces and produce finishes | COSVR67v3 | Edit 1 | 21/11 | Joe |

| Performance criteria Meet the contract specification | Knowledge and understanding P5 Meet the contract specification | Knowledge and understanding P3 Selection of resources |
| --- | --- | --- |
| P5 EDIT You must be able to: | K24 EDIT You need to know and understand: | K17 EDIT You need to know and understand: |
| comply with the contract information to apply solid render to background surfaces and produce finishes efficiently to the required specification by: - demonstrating work skills to: - measure - mark out - mix - apply - finish - using and maintaining hand and power tools, and equipment - applying base coats, reinforcing mesh and stress patches - preparing background surfaces, mixing and applying solid render to bellcasts, internal and external angles, walls, reveals and soffits, and to at least one of the following background surfaces to working instructions: - brick - block - concrete - rubble stone masonry - expanded metal lath (EML) - external insulation - producing a plain-faced finish coat to external walls and/or external insulation plus at least one of the following finishes to working instructions: - dash - rough-cast (harling, wet dash) - synthetic or non-synthetic renders - proprietary pre-cast | how to meet the contract specification in relation to the following: - how to carry out pre-installation checks to include structural integrity, dampness, vents, services (gas, electric, water, media cables) - how to mix render to the required strength for background surfaces and supporting fixtures - why it is necessary to prepare background surfaces - how to recognise the procedures to check flues and combustion air ventilation - why it is necessary to understand the implications of existing guarantees and warranties - how to apply base coats, reinforcing mesh and stress patches - how to apply multiple coat renders to external walls and external insulation - how to form internal and external angles, reveals, soffits, expansion joints and bellcasts - how to position, secure and apply renders to expanded metal lath (EML) - how to apply dash, plain-faced coat, rough-cast (harling, wet-dash), synthetic and non-synthetic renders and proprietary pre-cast finishes and sealants to external surfaces and external insulation including door and window reveals - how to complete post installation checks: compliance with specifications, resistance to water penetration, anchorage/fixing, vents, services (gas, electric, water, media cables) - how to recognise and determine when specialist skills and knowledge are required and report accordingly - the importance of recognising the limits of your own authority - the relevance of an assessment of significance - how to recognise specific requirements for structures of special interest, traditional construction, hard-to-treat buildings and historical significance - how to work with, around and in close proximity to plant and machinery - how to work at height using access equipment - how to use all hand and power tools and equipment - how and why operative care and maintenance of all hand and power tools and equipment is carried out | how the resources should be used and how any problems associated with the resources are reported in relation to: - render, sand, lime, cement and additives - bellcasts and beads, expanded metal lath (EML) - dash, rough-cast (harling, wet dash), proprietary pre-cast finish, synthetic and non-synthetic renders - reinforcement, stress patches, sealants, fixings and fittings - hand and power tools and equipment - digital equipment |
| Key Words |  |  |
| Renders; Bellcasts; Multiple-coat; Mixing renders; Expanded metal lath; Expansion-joints; Applying-renders; Internal-angles; External-angles; Dash finishes; Rough cast |  |  |
| Overview |  |  |
| This standard is about applying solid render to background surfaces and producing finishes by mixing and applying solid render to various surfaces and producing a plain-faced finish coat to external walls and external insulation to produce various finishes. This standard will also involve interpreting information, adopting safe, healthy and environmentally responsible work practices, selecting and using materials, components, tools and equipment, minimising damage, and working within the allocated time, in accordance with organisational requirements which are equal to or exceed current statutory and legislative requirements This standard is for people working in the occupational area of plastering and can be used by operatives, supervisors and managers |  |  |