| NOS Title | NOS Reference and New Version | Grid version control | Edit date | Edits by |
| --- | --- | --- | --- | --- |
| Produce complex external render finishes | COSVR77v3 | Edit 1 | 27/11 | Joe |

| Performance criteria Meet the contract specification | Knowledge and understanding P5 Meet the contract specification | Knowledge and understanding P3 Selection of resources |
| --- | --- | --- |
| P5 EDIT You must be able to: | K24 EDIT You need to know and understand: | K17 EDIT You need to know and understand: |
| comply with the contract information to produce complex external render finishes efficiently to the required specification by: - demonstrating work skills to: - measure - mark out - apply - finish two- and three-coat render - using and maintaining hand and power tools and, equipment - preparing background surfaces, mixing render and and producing four of the following external render finishes to working instructions: - tyrolean - dash - ashlar joint - rough cast (harling, wet dash) - scraped - textured - simulated stone - decorative | how to meet the contract specification in relation to the following: - why it is important to prepare background surfaces - how to mix sand, cement and lime based external renders - how to apply two- and three-coat rendering to ecternal solid backgrounds and expanded metal lath - how to form internal and external angles, reveals and expansion joints - how to form tyrolean, dash, ashlar joint, rough cast (harling, wet dash), scraped, textured, simulated stone and decorative render finishes - how to recognise and determine when specialist skills and knowledge are required and report accordingly - the importance of recognising limits of your own authority - the relevance of an assessment of significance - how to recognise specific requirements for structures of special interest, traditional construction, hard-to-treat buildings and historical significance - how to work with, around and in close proximity to plant and machinery - how to work at height using access equipment - how to use all hand and power tools and equipment - how and why operative care and maintenance of all hand and power tools and equipment is carried out | how the resources should be used and how any problems associated with the resources are reported in relation to: - render, sand, lime, cement and additives - clean water - hand and power tools and equipment digital equipment |
| Key Words |  |  |
| One-coat; Two-coat; Three-coat; Curved backgrounds; Inclined-surfaces; Mixing-plaster; Expansion-joints; Internal/external-angles; Expanded metal lath; Lath walls; Solid plaster; Complex surfaces |  |  |
| Overview |  |  |
| This standard is about applying solid plaster to complex internal surfaces by preparing background surfaces, mixing plaster and applying internal solid plaster to various surfaces This standard will also involve interpreting information, adopting safe, healthy and environmentally responsible work practices, selecting and using materials, components, tools and equipment, minimising damage, and working within the allocated time, in accordance with organisational requirements which are equal to or exceed current statutory and legislative requirements This standard is for people working in the occupational area of plastering and can be used by operatives, supervisors and managers |  |  |