| NOS Title | NOS Reference and New Version | Grid version control | Edit date | Edits by |
| --- | --- | --- | --- | --- |
| Repair complex decorative fibrous plaster components | COSVR83v3 | Edit 1 | 27/11 | Joe |

| Performance criteria Meet the contract specification | Knowledge and understanding P5 Meet the contract specification | Knowledge and understanding P3 Selection of resources |
| --- | --- | --- |
| P5 EDIT You must be able to: | K24 EDIT You need to know and understand: | K17 EDIT You need to know and understand: |
| comply with the contract information to repair complex decorative fibrous plaster components efficiently to the required specification by: - demonstrating work skills to: - survey - measure - mark out - remove - replicate - fix - position - secure - finish - using and maintaining hand and power tools and equipment - preparing background surfaces, mixing casting plaster, taking thixotropic squeezes (impressions) and repairing one of the following complex decorative fibrous plasterwork components to working instructions: - cornices - dados - skirtings - panels tying-back fibrous plaster components to structure | how to meet the contract specification in relation to the following: - how to prepare background surfaces - how to mix casting plasters - how to take impressions, including thixotropic squeezes - how to repair complex decorative fibrous plaster components - how tie-back fibrous plaster components to structure - how to replicate mouldings - why it is important to reinforce around damaged areas - why it is important to recognise and determine when specialist skills and knowledge are required and report accordingly - the importance of recognising the limits of your own authority - the relevance of an assessment of significance - how to recognise specific requirements for structures of special interest, traditional construction, hard-to-treat buildings and historical significance - how to work with, around and in close proximity to plant and machinery - how to use all hand and power tools and equipment - how to work at height using access equipment how and why operative care and maintenance of all hand and power tools and equipment is carried out | how the resources should be used and how any problems associated with the resources are reported in relation to: - casting plasters, reinforcing material, timber and fixings - release agents - thixotropic rubber - clean water - hand and power tools and equipment - digital equipment |
| Key Words |  |  |
| Replicate-mouldings; Plasterwork; Casting-plasters; Repair-cornices; Repair-dados; Reinforce-joints; Thixotropic |  |  |
| Overview |  |  |
| This standard is about preparing complex decorative fibrous plaster components by preparing background surfaces, mixing casting plaster, taking thixotropic squeezes (impressions), repairing plasterwork, and tying-back fibrous plaster components to structures This standard will also involve interpreting information, adopting safe, healthy and environmentally responsible work practices, selecting and using materials, components, tools and equipment, minimising damage, and working within the allocated time, in accordance with organisational requirements which are equal to or exceed current statutory and legislative requirements This standard is for people working in the occupational area of plastering and can be used by operatives, supervisors and manager |  |  |