| NOS Title | NOS Reference and New Version | Grid version control | Edit date | Edits by |
| --- | --- | --- | --- | --- |
| Run in-situ mouldings | COSVR80v3 | Edit 1 | 27/11 | Joe |

| Performance criteria Meet the contract specification | Knowledge and understanding P5 Meet the contract specification | Knowledge and understanding P3 Selection of resources |
| --- | --- | --- |
| P5 EDIT You must be able to: | K24 EDIT You need to know and understand: | K17 EDIT You need to know and understand: |
| comply with the contract information to run in-situ mouldings efficiently to the required specification by: - demonstrating work skills to: - measure - mark out - fit - apply - run - position - secure - using and maintaining hand and power tools and equipment - preparing backgrounds and moulds, gauging and mixing materials and running in-situ mouldings straight and/or curved to working instructions for any one of the following: - cornices - dados - skirting - panels - angles - arches forming joints; mitres; returns; stop-ends; short breaks | how to meet the contract specification in relation to the following: - how to produce templates and construct running moulds - prepare backgrounds, set out and run in-situ straight and curved mouldings for cornices, dados, skirting, angles, panels and arches - how to prepare, gauge and mix materials - how to form short breaks and returns, short lengths and returns, joints and mitres - how to set out and fix running rules in situ, including overlaps - how to reproduce shape of existing mould to form template - core-out moulding - why it is important to prevent build-up and gathering-on - how to recognise and determine when specialist skills and knowledge are required and report accordingly - the importance of recognising the limits of your own authority - the relevance of an assessment of significance - how to recognise specific requirements for structures of special interest, traditional construction, hard-to-treat buildings and historical significance - how to work with, around and in close proximity to plant and machinery - how to use all hand and power tools and equipment - how to work at height using access equipment - how and why operative care and maintenance of all hand and power tools and equipment is carried out | how the resources should be used and how any problems associated with the resources are reported in relation to: - timber, timber-based products, sheet materials, metal laths, sand, cement, lime, plaster - bonding agents, additives - clean water - fixings and fittings - hand and power tools and equipment digital equipment |
| Key Words |  |  |
| Run in-situ mouldings; Core-out; Plasters; Running curved-cornices; Arches; Panel mouldings; Plasterwork; Lime; Cement; Dado; Skirting |  |  |
| Overview |  |  |
| This standard is about running in-situ mouldings by preparing and running straight and curved in-situ mouldings in various formats This standard will also involve interpreting information, adopting safe, healthy and environmentally responsible work practices, selecting and using materials, components, tools and equipment, minimising damage, and working within the allocated time, in accordance with organisational requirements which are equal to or exceed current statutory and legislative requirements This standard is for people working in the occupational area of plastering and can be used by operatives, supervisors and managers |  |  |